Cisco UCS C220 M5SX Physical build

Enter servers IP address in browser “https://10.91.6.3”

You will be prompted for login details, use the following

admin

password

Change to:

ILOAdmin

IL0Adm1n

*Check firmware version - if its lower than 4.0(4f) then upgrade it.

Updating Firmware version

Launch KVM (top right) > Select HTML Based KVM

Click on KVM Console

Select Activate virtual media and browse to the location (Z:\\emeaapprepository\CiscoUCSFirmware\UCS C220 or "Z:\UCS C220 M5\Host Upgrade Utility\ucs-c220m5-huu-4.0.4f.iso"

M5\Host Upgrade Utility) that contains the update file and ok.

Then reboot server and press F6 to get into Boot Menu > Choose DVD (TBC) > Select I agree

Select all the items under components > Select all options under the Storage Drives > press Update and Yes to update the selected components.

Once complete, select Update

Do you wish to proceed, select YES

Once this stage is complete you will be prompted Update and Activate All

Do you want to proceed with Firware update, select Yes

Reverting to the Previous Firmware Version

There may be instances when you might have to revert to the previously installed firmware version.After installing and activating a later version, you can revert to the previous version using the following options in the CIMC Web UI:

Go to Navigation > Admin > Firmware Management

RAID Configuration

The server BIOS includes a configuration utility specific to this controller that you can use to create and

delete RAID volumes, view controller properties, and erase the physical drive contents. Access the utility

by pressing F2 when prompted during server boot. Then navigate to Advanced > Avango MegaRAID  > Configure

Create Virtual Drive >

Enter to select drive and enable, repeat the same process to enable the other drives and OK to commit the changes

Highlight Confirm and press enter > Enable > select Yes to proceed

Select RAID level - enter > select desired RAID level - enter

Move down to Select drives from - enter and ensure Unconfigured Capacity is selected

Select Drive Groups > Choose Drive Group > Drive Group 0: RAID1 > Select Enable > Apply Changes > OK

Configure Virtual Drive Parameters and select Virtual Drive Size:

Select the size of your Virtual disk > Default Initialisation and select Full > Save Configuration

And repeat process for additional virtual drives

*For file systems

\data

\opt

Select Drives - enter (as shown below) > Choose Unconfigured drives and select each disk and enable > Apply changes - enter

A message will appear that all free space has been used.

OK - enter > go to Save Configuration - enter

Select confirm and enable - enter > select yes - enter > ok

F10 save and reset

Raid Configuration using the CIMC (GUI)

Virtual Drive:

Physical Drive:

Select both physical drives and click chevron tab to unconfigure from JBOD

Select ”Create Virtual Drive form Unused Physical Drives” (will be RAID1 in next screen)

Linux 7.6 Build

Launch the KVM from the CIMC console

Select HTML KVM

Select Virtual Media and activate virtual devices

Select virtual Media again and select Map CD\DVD (as shown below)

When clicking on Browse, it will display network drives where Full Satellite iso was downloaded to on your PC  > open > map drive Select Power and power cycle the server, press F6 to enter the Boot Menu

Select VKVM-Mapped VDVD and enter

Select Satellite Full ISO build that you have created and placed on your C: drive Press any key to boot from DVD

Once the OS install has completed, from the KVM > select Macros > Static Macro > Select CTRL ALT Del

Set password and login